# CS-350  Emerging System Architecture And Technology

# Module 2

# Milestone One

# Pulse Width Modulation (PWM) Lab

Submitted By:

HAMNA KHALID

ID: 2902671

hamna.khalid@snhu.edu

Submitted To:

PROFESSOR CHRIS KARDELL

c.kardell@snhu.edu

Southern New Hampshire University

Wednesday, June 10, 2026

# Module Two

# Milestone One

# Pulse Width Modulation (PWM) Lab

# Introduction

Embedded systems require precise control over hardware components, and Pulse Width Modulation (PWM) is one of the most important techniques for achieving this control (Raspberry Pi Foundation, n.d.). In this lab, PWM was implemented on a Raspberry Pi to control an LED's brightness. The purpose of this milestone was to demonstrate how digital signals can simulate analog behavior by varying the duty cycle of a signal.

This lab builds on prior work with basic LED control and introduces more advanced timing and signal manipulation concepts. The implementation and analysis in this lab were guided by the Milestone One PWM lab instructions provided in the course materials (Southern New Hampshire University, n.d.).

# Concept Overview: Pulse Width Modulation (PWM)

Pulse Width Modulation (PWM) is a technique used in embedded systems to control power delivery without changing voltage levels (Kardell, n.d.). It is also widely used in electronic signal control and hardware interfacing applications (Horowitz & Hill, 2015). Instead of providing a constant voltage, the signal rapidly switches between ON and OFF states.

The key concept behind PWM is the duty cycle, which represents the percentage of time the signal remains ON during each cycle. For example:

- 0% duty cycle → LED OFF

- 50% duty cycle → LED partially bright

- 100% duty cycle → LED fully ON

By adjusting the duty cycle, the brightness of the LED can be controlled smoothly, creating the appearance of an analog output.

# System Design And Implementation

The system consists of a Raspberry Pi, an LED, a resistor, and jumper wires connected through a breadboard. GPIO pin 18 was configured as the output pin for PWM control. The hardware setup and LED circuit configuration were reinforced using external instructional resources to ensure proper wiring and safe operation (O’Reilly, 2013).

Additionally, Raspberry Pi GPIO functionality enables flexible interaction between software and physical hardware components (Monk, 2016).

The circuit follows a simple path:

GPIO18 → Resistor → LED → Ground (GND)

The resistor is used to limit current and protect the LED from damage.

# Code Implementation (Best Practices Applied)

The following Python code was developed using proper formatting, commenting, and logical structure:

import RPi.GPIO as GPIO import time # Use Broadcom pin numbering GPIO.setmode(GPIO.BCM) # Set GPIO18 as output GPIO.setup(18, GPIO.OUT) # Create PWM instance at 100 Hz pwm = GPIO.PWM(18, 100) # Start PWM at 0% duty cycle pwm.start(0) try: while True: # Increase brightness for duty in range(0, 101, 5): pwm.ChangeDutyCycle(duty) time.sleep(0.1) # Decrease brightness for duty in range(100, -1, -5): pwm.ChangeDutyCycle(duty) time.sleep(0.1) # Handle keyboard interrupt safely except KeyboardInterrupt: pass # Clean up GPIO resources pwm.stop() GPIO.cleanup()

# Functionality And Behavior Analysis

The LED gradually increases in brightness and then decreases, creating a smooth fading effect. The complete PWM cycle, from minimum brightness to maximum brightness and back to minimum, takes approximately 4.58 seconds.

This demonstrates controlled and gradual variation in duty cycle, resulting in a smooth transition of LED brightness. The observed timing reflects the delay intervals and incremental duty cycle changes defined in the program, confirming that the PWM signal is functioning as intended.

# Frequency And Duty Cycle Analysis

At higher frequencies, the LED appears stable because the switching occurs too quickly for the human eye to detect (Raspberry Pi Foundation, n.d.). This aligns with standard electronic principles of signal perception and timing behavior (Horowitz & Hill, 2015).

The duty cycle was adjusted in increments of 5%, producing a smooth visual transition. Smaller increments could further improve smoothness, while larger increments would make brightness changes more noticeable.

# Lab Questions

## Question No 1

At What Frequency Can You See The LED Start To Blink?

## Answer

The LED begins to visibly blink when the PWM frequency is reduced to approximately 50–60 Hz or lower. At higher frequencies, the switching occurs too rapidly for the human eye to detect, resulting in a steady light output.

## Question No 2

At What Duty Cycle Is The Intensity Of The LED Perceptibly Diminished From The Initial 50% Duty Cycle?

## Answer

The LED’s brightness becomes noticeably dimmer when the duty cycle is reduced below approximately 40–45%. At 50%, the LED appears moderately bright, but decreasing the duty cycle further produces a perceptible reduction in intensity.

## Question No 3

Was The Transition Smooth? How Could It Be Improved? Why?

## Answer

The transition was smooth, using 5% increments every 0.1 seconds. However, smoother transitions could be achieved by reducing the increment size (e.g., 1–2%) or decreasing the delay interval. Smaller changes in duty cycle produce finer brightness adjustments, which appear smoother to the human eye.

## Question No 4

What Function Sets The PWM Frequency For A GPIO Line?

## Answer

The PWM frequency is set using:

GPIO.PWM(pin, frequency)

In this lab:

GPIO.PWM(18, 100)

## Question No 5

What Function Sets The Duty Cycle For A GPIO Line?

## Answer

The duty cycle is controlled using:

pwm.ChangeDutyCycle(value)

This function adjusts the percentage of time the signal remains ON.

# Applications Of PWM In Embedded Systems

PWM is widely used in embedded systems and electronics for controlling hardware components efficiently (Kardell, n.d.; Horowitz & Hill, 2015).

- LED Brightness Control

- Motor Speed Control

- Servo Motor Positioning

- Power Regulation

- Signal Modulation

# Challenges And Improvements

One of the primary challenges encountered during this lab was ensuring stable hardware connections between the Raspberry Pi and the breadboard components. Proper pin alignment and secure wiring were essential for reliable operation.

Future improvements could include:

- Using smaller duty cycle increments for smoother transitions.

- Implementing user-controlled brightness.

- Expanding the system to control multiple LEDs.

# Conclusion

This lab successfully demonstrated the implementation of PWM on a Raspberry Pi to control LED brightness. By varying the duty cycle of a digital signal, analog-like behavior was achieved. PWM is a fundamental concept in embedded systems and provides a foundation for more advanced hardware control applications.This implementation demonstrates both functional correctness and practical application of PWM concepts in embedded system design.

# AI Usage Acknowledgment

AI tools were used to assist in structuring the document, refining technical explanations, and ensuring alignment with rubric requirements. All implementation, testing, and final submission decisions were completed by the student.

# Acronyms And Full Forms

- GND – Ground

- GPIO – General Purpose Input/Output

- PWM – Pulse Width Modulation

# References

- Horowitz, P., & Hill, W. (2015).

The art of electronics (3rd ed.).

Cambridge University Press.

- Kardell, C. (n.d.).

Emerging systems architecture and technologies.

Southern New Hampshire University.

- Monk, S. (2016).

Programming the Raspberry Pi: Getting started with Python.

McGraw-Hill Education.

- O’Reilly. (2013, December 11).

Connecting an LED to Raspberry Pi.

YouTube. https://www.youtube.com/watch?v=BWYy3qZ315U

- Raspberry Pi Foundation. (n.d.).

GPIO and PWM documentation.

https://www.raspberrypi.org/documentation/

- Southern New Hampshire University. (n.d.).

CS-350 Milestone One PWM lab guide.

Brightspace